J8L Application Guide

Ver. 1.1 For J8L firmware ver. 0.3.8

Date. 27, 09, 2024

Supported Command List

| Command | Description | Arguments | Return Value | Notes | Functional Verification |
| --- | --- | --- | --- | --- | --- |
| ToF Commands |  |  |  |  |  |
| settofconf | Turns on the ToF sensor | N/A | OK/NG Note 1 |  | OK |
| settofpwr | Turns off the ToF sensor | 0 | OK/NG Note 1 | Interrupts are disabled, but data continues | OK |
| Set Data Commands |  |  |  |  |  |
| setlcdinvl | Set left LCD panel inversion | 0-3 (Inversion directions) | OK/NG Note 1 | 0: Left/Up (default) 1: Right/Up 2: Left/Down 3: Right/Down | OK |
| setlcdinvr | Set right LCD panel inversion | 0-3 (Inversion directions) | OK/NG Note 1 | 0: Left/Up (default) 1: Right/Up 2: Left/Down 3: Right/Down | OK |
| setlcdluxl | Set left LCD panel luminance | 1000-5000 (Multiples of 100) | OK/NG Note 1 | Luminance must be a multiple of 100 | OK |
| setlcdluxr | Set right LCD panel luminance | 1000-5000 (Multiples of 100) | OK/NG Note 1 | Luminance must be a multiple of 100 | OK |
| setlcdhorl | Set left LCD horizontal orbit | +: 000-016, -: 100-116 | OK/NG Note 1 | 0 = positive orbit 1 = negative orbit | OK |
| setlcdhorr | Set right LCD horizontal orbit | +: 000-016, -: 100-116 | OK/NG Note 1 | 0 = positive orbit 1 = negative orbit | OK |
| setlcdvorl | Set left LCD vertical orbit | +: 000-016, -: 100-116 | OK/NG Note 1 | 0 = positive orbit 1 = negative orbit | OK |
| setlcdvorr | Set right LCD vertical orbit | +: 000-016, -: 100-116 | OK/NG Note 1 | 0 = positive orbit 1 = negative orbit | OK |
| setlcddefault | Set all LCD setting to default value | N/A | OK/Error | Resets all settings stored in flash | OK |
| setlcdmode | Set display mode (60/120Hz, 2D/3D) | 0: 60Hz/2D 1: 120Hz/2D 2: 60Hz/3D 3: 120Hz/3D | OK/NG Note 1 | Wait for device stability before switching modes | OK |
| setlcddevicel | Set left panel serial number | String (< 18 characters) | OK/Error | Serial number must be 18 characters or less | OK |
| setlcddevicer | Set right panel serial number | String (< 18 characters) | OK/Error | Serial number must be 18 characters or less | OK |
| Get Data Commands |  |  |  |  |  |
| getfirmver | Get the firmware version | N/A | Firmware version |  | OK |
| getbrgfw | Get the firmware version of the bridge | N/A | Firmware version |  | OK |
| getlcdvalue | Get current LCD settings | N/A | LCD setting | Shows the current LCD settings | OK |
| Flash Commands |  |  |  |  |  |
| flashr | Read LCD settings from flash | N/A | OK/Error | Replaces current LCD settings with those from flash | OK |
| flashw | Write current LCD settings to flash | N/A | OK/Error | Saves current LCD settings to flash | OK |
| Special Commands |  |  |  |  |  |
| enterbootloader | Enter bootloader mode | N/A | Not Implemented | Immediately switches the system to DFU mode | OK |

Note 1: If the message returns "NG", an error code will follow: 0 = Invalid Command, 1 = Invalid Parameter, 2 = I/O Error, 3 = Command Not Allowed (Incorrect System State)

Button Event List

| Event Type | Description | Action/Effect | Return Value | Example | Functional Verification |
| --- | --- | --- | --- | --- | --- |
| volume | Triggered when BTN_VOL_PLUS or BTN_VOL_MINUS is pressed | Adjusts the speaker volume | +1/-1 Note 1 | #volume +1@ | OK |
| lcdmode | Triggered when BTN_LCD_MODE is pressed | Displays the current LCD mode after the mode is changed | 0, 2 Note 1,2 | #lcdmode 2@ | OK |

Note 1: The returned CDC events are prefixed with the ‘#’ symbol and end with the ‘@’ symbol.

Note 2: For lcdmode, the value alternates between 0 (60Hz/2D) and 2 (60Hz/3D), as 120Hz modes have been removed.